Rules for Making Bode Plots

| # Term | Magnitude | Phase |
| --- | --- | --- |
| Constant: K | 20·log10(¦K¦) | K>0: 0° K<0: ±180° |
| Real Pole: | - Low freq. asymptote at 0 dB - High freq. asymptote at -20 dB/dec - Connect asymptotic lines at 0, | - Low freq. asymptote at 0°. - High freq. asymptote at -90°. - Connect with straight line from 0.1·0 to 10·0. |
| Real Zero*: | - Low freq. asymptote at 0 dB - High freq. asymptote at +20 dB/dec. - Connect asymptotic lines at 0. | - Low freq. asymptote at 0°. - High freq. asymptote at +90°. - Connect with line from 0.1·0 to 10·0. |
| Pole at Origin: | - -20 dB/dec; through 0 dB at =1. | - Line at -90° for all . |
| Zero at Origin*: s | - +20 dB/dec; through 0 dB at =1. | - Line at +90° for all . |
| Underdamped Poles: | - Low freq. asymptote at 0 dB. - High freq. asymptote at -40 dB/dec. - Connect asymptotic lines at ω0. - Draw peak† at freq= ω0, with amplitude H(jω0)=-20·log10(2ζ) | - Low freq. asymptote at 0°. - High freq. asymptote at -180°. - Connect with line from ω=ω0·10-ζ to ω0·10ζ |
| Underdamped Zeros*: | - Low freq. asymptote at 0 dB. - High freq. asymptote at +40 dB/dec. - Connect asymptotic lines at ω0. - Draw dip† at freq= ω0, with amplitude H(jω0)=+20·log10(2ζ) | - Low freq. asymptote at 0°. - High freq. asymptote at +180°. - Connect with line from ω=ω0·10-ζ to ω0·10ζ |
| Time Delay: | - No change in magnitude | - Phase drops linearly. Phase = -ωT radians or -ωT·180/π°. On logarithmic plot phase appears to drop exponentially. |
| Notes: ω0 is assumed to be positive. If ω0 is negative, magnitude is unchanged, but phase is reversed. * Rules for drawing zeros create the mirror image (around 0 dB, or 0) of those for a pole with the same 0. † We assume any peaks for ζ>0.5 are too small to draw, and ignore them. However, for underdamped poles and zeros peaks exists for 0<ζ<0.707=1/√2 and peak freq. is not exactly at, 0 (peak is at ). For nth order pole or zero make asymptotes, peaks and slopes n times higher than shown. For example, a double (i.e., repeated) pole has high frequency asymptote at -40 dB/dec, and phase goes from 0 to –180o). Don’t change frequencies, only the plot values and slopes. |  |  |

Tool for learning how to draw Bode plots by hand at:   https://lpsa.swarthmore.edu/Bode/bodeDraw.html

Quick Reference for Making Bode Plots

If starting with a transfer function of the form (some of the coefficients bi, ai may be zero).

Factor polynomial into real factors and complex conjugate pairs (p can be positive, negative, or zero; p is zero if a0 and b0 are both non-zero).

Put polynomial into standard form for Bode Plots.

Take the terms (constant, real poles and zeros, origin poles and zeros, complex poles and zeros) one by one and plot magnitude and phase according to rules on previous page.  Add up resulting plots.